# SARA GRESHAM, CRC 1331978

# 386-365-8685

# NORRIS.SARA2011@GMAIL.COM

04/05/2023

Job: Morrison Bathroom Remodel

Description: Converting current ½ and extra master closet into a full bath

Builder: responsible for removing the non-bearing 2x4 wall between bath and closet. Remove the 2’6 door from master bedroom side of closet and frame up wall to be drywalled. Then reframe a 2’6 door opening on opposite closet wall from child’s bedroom. Responsible for handling drywall, painting, and trim base.

Plumbers: move plumbing pipes and drain lines for toilet and sink. Add pex line and drain line for new tub. Plumbers to install new plumbing fixtures.

Electricians: moving current electrical out of wall separating two rooms, moving an outlet in child’s room to make room for the 2’6 door; running new line and switches for vanity light, tub can light/ exhaust fan, and outlet at vanity. Electrician to install new electrical fixtures.

Tile/ Flooring company: Patching/ leveling floor, replacing current flooring with new tile, tiling tub surround.

Cabinet Company: replacing vanity base with 48in cabinet; new Corian top with undermount sink.

Current interior doors will be used.